MATERIAL SAFETY   DATA    SHEET

## TOP 66

1.	IDENTIFICATION OF PRODUCT & COMPANY

# Product name	                         TOP 66

Product Use		Foam degreaser for aluminum

Date of version		12.11.2013

Company	ZOHAR DALIA

Address	19239 Kibbutz Dalia, Israel

Phone	972-4-9897234

Fax	972-4-9897200

2. 	HAZARD IDENTIFICATION

Physical state	Liquid (liquid)

Emergency overview	CAUTION !

causes severe burns

Avoid contact with eyes and skin.

Potential acute health effects:

Eyes	causes severe burns to eyes

Skin	causes burns

Inhalation	Moderate hazardous in case of inhalation

Ingestion	Moderate hazardous in case of ingestion

3.	COMPOSITION / INFORMATION ON INGREDIENTS

| Material | Cas No. | Symbol | R Phrase | Content |
| --- | --- | --- | --- | --- |
| Sodium hypochlorite 10% | 7681-52-9 | C | R31, R34 | >30% |
| Sodium silicate | 1344-09-8 | Xi | R36/38 | 1 – 5% |
| Sodium hydroxide | 1310-73-2 | C | R35 | 1 – 5% |
| Alkyl dimethyl amine oxides | 61788-90-7 | Xi, N | R38, R41, R50 | 1 – 5% |
| 2-phosphonobutane-1,2,4-tricarboxilic acid | 37971-36-1 | C | R34 | 1 – 5% |

4.	FIRST AID MEASURES

Inhalation	if inhaled, remove to fresh air. Consult a doctor.

Skin contact	Rinse with plenty of water and soap.

Eye contact	Wash with plenty of water, see an oculist

Ingestion		Wash out with water, drink Water for stomach dilution. Don’t induce vomiting. Seek immediately medical attention.

5.	FIRE FIGHTING MEASURES

Auto ingestion temperature	Not applicable

Flash point	Not applicable

Flammable limits	Not flammable

Hazardous thermal decomposition products	Decomposition products may include the

following materials:

Carbon dioxide

Carbon monoxide

Sulfur oxides

Fire fighting media and instructions	Use an extinguishing agent suitable for the

surrounding fire. Dyke area of fire to prevent runoff.			No specific fire or explosion hazard.

Special protective equipment for fire fighters	Fire fighters should wear appropriate protective

equipment and self contained breathing apparatus

with a full face piece operated in positive pressure

mode.

6.	ACCIDENTAL RELEASE MEASURES

Personal precautions	Wear protective equipment

Keep unprotected person away

Environmental precautions	Do not allow to flow into drainage system.

Methods for cleaning up	Remove with liquid – absorbing material (sand,peat,sawdust).

7.	HANDLING AND STORAGE

Handling 	When diluting, always stir the product into standing water.

Open and handle container with care.

Storage	Store protected from light.

Do not store together with edibles or other consumble substance

Do not keep together with acids in one container

Storage class	VCI – storage class: 8B

8.	EXPOSURE CONTROLS / PERSONAL PROTECTION

General protective measures	Ensure for good ventilation/suction.

Hand protection	Wear protective gloves.

Eye  protection	Use goggles which can be tightly sealed .

Respiratory protection	If there is atomization

9.	PHYSICAL AND CHEMICAL PROPERTIES

Appearance	Clear pale yellow liquid

Odor	of chlorine

pH 2%	12.5 - 13

Flash point	 Aqueous preparation

Relative density  gr/ml 20°C	1.12

10.	STABILITY AND REACTIVITY

Conditions to avoid	No decomposition if used according to specification.

Materials to avoid	React with acids: heat generated and chlorine released.

Hazardous decomposition            	At higher temperature chlorine may be generated.

11.	TOXICOLOGICAL INFORMATION

Routes of Entry: Absorbed through skin. Eye contact. Inhalation. Ingestion.

Toxicity to Animals: Acute oral toxicity (LD50): 5800 mg/kg [Mouse]. (Sodium hypochlorite).

Chronic Effects on Humans:

CARCINOGENIC EFFECTS: Classified 3 (Not classifiable for human.) by IARC [Sodium hypochlorite]. MUTAGENIC EFFECTS: Mutagenic for bacteria and/or yeast. [Sodium hypochlorite]. Mutagenic for mammalian somatic cells. [Sodium hydroxide]. Contains material which may cause damage to the following organs: lungs, mucous membranes, skin, eyes.

Other Toxic Effects on Humans: Very hazardous in case of skin contact (irritant), of ingestion, . Hazardous in case of skin contact (corrosive), of eye contact (corrosive). Slightly hazardous in case of inhalation (lung sensitizer, lung corrosive).

Special Remarks on Toxicity to Animals: Not available.

Special Remarks on Chronic Effects on Humans: May affect genetic material (mutagenic) (Sodium hypochlorite)

Special Remarks on other Toxic Effects on Humans:

Potential Health Effects: Can cause severe irritation and possible burns to skin and eyes. Eye contact may also cause corneal and conjunctival edema, conjunctival hemorrhages. Contact with skin may also cause vesicular eruptions and eczematoid dermatitis which becomes evident upon re-exposure. Prolonged or repeated eye contact may cause conjunctivitis. Ingestion can cause burns to the digestive tract. Symptoms may include: 1. pain and inflammation of the mouth, pharynx, esophagus, and stomach, 2. erosion of the mucous membranes (chiefly of the stomach), nausea, vomiting, choking, coughing, hemorrhage, 3. circulatory collapse with cold and clammy skin (due to methemoglobinemia), cyanosis, and shallow respirations, 4. confusion, delirium, coma, 5. edema of the pharynx, glottis, larynx with stridor and obstruction, 6. Perforation of the esophagus, or stomach, with mediastinitis or peritonitis. Inhalation causes slight to severe respiratory tract irritation and delayed pulmonary edema. Prolonged or repeated inhalation may cause allergic respiratory reaction (asthma).

12. 	ECOLOGICAL INFORMATION

Persistence / Degradability	the product contains active chlorine or active chlorine carriers,  				respectively. Locally set limits in sewage have to be observed.

In case of discharge of acidic or alkaline products into the

sewerage system the pH must be in the range of 6-10 since

higher or lower pH values may damage the sewerage system or

impair biological sewage treatment plant. Local discharge

statutes have to be observed.

Ecotoxicity	The product is not subject to ecological classification

according to directive 1999/45/EC

13.	DISPOSAL CONSIDERATIONS

Dispose of according to local authority regulations

14.	TRANSPORT INFORMATION

UN No	1791

ADR / RID	8 HYPOCHLORIT SOLUTION

ICAO/IATA	8 HYPOCHLORIT SOLUTION

IMDG	8 HYPOCHLORIT SOLUTION

Packing group III

15.	REGULATORY INFORMATION

NON EDIBLE

Classifications/symbols	C Corrosive

Risk phases	R31: Contact with acids liberates toxic gas

R35: Causes sever burns

Safety phases	S26: In case of contact with eyes, rinse immediately with plenty of  water and seek for medical advice.

S36/37/39: Wear suitable protective clothing, gloves and

eye/ face protraction.

S45: In case of accident or if you feel unwell, seek for medical

advice  immediately.(show label where possible)

S50: Do not mix with acids

16.	OTHER INFORMATION

This information is based on our present knowledge. However, this shall not constitute a guarantee for any specific product features and shall not establish a legally valid contractual relationship.

Full text of R phrases indicated by codes in the safety data sheet.

R31: Contact with acids liberates toxic gas

R34: Causes burns

R35: Causes severe burns

R36: Irritating to eyes

R41: Risk of series damage to the eyes

* All ZOHAR DALIA  products are KOSHER PARVE. Relevant documentation will be supplied as requested.

# DISCLAIMER

The information contained in this document is given in good faith and based on our current knowledge. It is only an indication and is in no way binding, notably as regards infringement of or prejudios to third parties through the use of our products.

ZOHAR DALIA GUARANTEES THAT ITS PRODUCTS COMPLY WITH ITS SALES SPECIFICATIONS.

This information must on  no account be used as a substitute for necessary prior tests which alone can ensure that a product is suitable for a given use.

Users are responsible for ensuring compliance with local legislation and for obtaining the necessary certifications and authrisations.

Users are requested to check that they are in possession of the latest version of the present document and ZOHAR DALIA is at their disposal to supply any additional information.